I’m interested in your services to create a technical prototype for a water control device I've developed. The device is functional but requires optimization in terms of size and integration, as it's currently too large due to the tubing and component arrangement.

Here’s a detailed overview of the device's functionality and requirements:

Current Device Setup:

- Location: Installed under my kitchen sink.

- Functionality:

  - Connected to the cold water valve, with water flow directed through two pathways using a magnetic valve system controlled by a switch.

  - When the switch is toggled to the left, the left valve opens, allowing water to flow through a filter, while the right valve remains closed. Conversely, when toggled to the right, the water bypasses the filter, and the right valve opens.

  - A water flow sensor behind the filter, connected to a microcontroller, monitors the flow and sends a warning email when the filter reaches 1000 liters of use.

- Purpose:

  - To provide filtered drinking water through the main kitchen tap without the need for a separate tap.

  - To avoid filtering all water usage to preserve water pressure and filter longevity.

What I Need:

- Size Optimization:

  - I’m looking for ideas on how to reduce the device's size significantly. Currently, it takes up too much space because of the hoses connecting all parts.

  - Ideally, I'd like everything integrated into a compact box.

- Technical Requirements:

  - The box should accommodate:

    - Two Magnetic Valves (around 9x7x4 cm each), though I’m open to alternative suggestions if they offer faster switching than three-way valves, which tend to be slower (5-15 seconds).

    - A Microcontroller (approx. 4x3x0.7 cm).

    - Tubing to connect all elements properly.

  - The box should feature:

    - An inlet for cold water.

    - Outlets for:

      - Unfiltered water going to the filter.

      - Filtered water returning from the filter.

      - A bypass for unfiltered water.

Request:

Could you provide an estimate or price quote for designing a compact version of this device, possibly with suggestions on component optimization?

Thank you, and I look forward to discussing this further.

Best regards,
Martin Altmann